XXXIII CONFERENZA ITALiANA DI SCIENZE REGIONALI

EFFECTIVENESS OF EUROPEAN EXPERIENCES IN URBAN FREiGHT TRansport

Andrea CAMPAGNA, Paolo DELLE SITE,  Francesco FILIPPI, Christopher ROWLAND

ABSTRACT

The paper presents a review of the existing practices and measures that are employed by both the public and private sectors to maximise the effectiveness of the urban section of the freight transport chain across the Member States of the European Union and achieve “sustainable urban distribution,” which combines maximising the economic efficiency of distribution in urban areas, while minimising the environmental and social impacts. Specific solutions applied or piloted locally are investigated, with particular reference to innovative schemes which are able to improve the efficiency of freight distribution in cities and, at the same time, achieve environmental sustainability.  Solutions are characterised in terms of both their effectiveness with respect to the intended objectives and their practical feasibility.  The research study upon which this paper is based was commissioned by DG MOVE of the European Commission and concluded with some recommendations on potential initiatives that could be taken at a European level to facilitate the development of sustainable urban distribution.

- Introduction

In the literature there is no common definition of Urban Freight Transport (UFT), with various different terms being used, such as “city logistics”, “urban distribution” and “urban goods movement” to refer to the same general concept of the transportation of goods within urban areas. Freight movements in urban areas can be classified according to both the type of vehicle (generally determined by the legal classifications used by vehicle licensing regimes in different countries) and/or the purpose of the trip being made.

In this paper we have chosen to define UFT as “the movement of freight vehicles whose primary purpose is to carry goods into, out of and within urban areas”.  This definition excludes, for example, shopping trips in passenger vehicles and movements of “white vans” for servicing and maintenance.

The definition of urban area used in this paper is recommended by the United Nations and is as follows:  “Areas that have urban (i.e. built-up) land of 20 or more hectares that are less than 200 metres apart and linked to form a continuous built-up area”.  This provides a reasonable definition for the purposes of this study, but it includes a very wide range of towns and cities with different freight transport issues at varying levels of intensity.  Therefore it has to be recognised that not all the measures and practice that can contribute to imporved effectiveness in UFT will be appropriate in all urban area, which can vary from ”metropolises” with large populations very significant road congestion and air quality issues to small rural towns that may experience relatively insignificant road congestion at peak hours but may have no air quality issues.

The key issue in terms of “effectiveness” of UFT revolves around how to balance the needs of urban economies with the externalities generated by movements of freight vehicles.  UFT is essential to the functioning of urban economies as it is required, for example, to replenish stocks of food and other retail goods in shops, to deliver documents, parcels and other supplies to offices and to remove household waste.  UFT can also be inefficient from an operational point of view and the additional costs associated with this inefficiency are generally passed on by freight transport operators to their customers and ultimately onto final consumers who live and work in the urban area. Inefficiency in distribution in urban areas can be exhibited in the following ways:

- Low load factors for freight vehicles and empty running;

- A high number of deliveries made to individual premises within a given time period;

- Long dwell times at loading and unloading points.

However, shippers, receivers and their transport operators do not always have a significant incentive to increase the efficiency of the deliveries to reduce costs.  This is because the transport cost is often only a small proportion of the value of the goods that are being transported and the overall costs of their customers.

Although UFT performs an  important role in the economic welfare of cities and therefore supports urban economies, it also has a number of negative effects that are not fully internalised into the costs borne by the market, such as the impact of freight vehicles on urban and inter-urban road congestion, reducing air quality (particularly as almost all freight vehicles are diesel-powered), emitting greenhouse gases (GHG) and creating noise (which has a particular impact on the potential for night-time deliveries close to residential areas).

In economic terms, these negative impacts are due to the lack of internalisation by the users and operators of UFT services of the total external costs associated with urban freight transport; in other words, not all of the social costs are reflected in the price of freight transport charged by the freight operators to their customers. Therefore, despite the economic benefits generated by UFT, there is a justification for intervention by the public sector in the market to redress the balance between social cost and social benefit derived from UFT.

While intervention by the public sector is justified in order to reduce the negative impacts of UFT, the most effective measures at all levels of policy-making are most likely to be those that also incentivise UFT operators and/or their customers to increase distribution efficiency.  Such measures will have the effect of both reducing costs and/or adding value for the private sector while also reducing negative externalities.  In this paper it is argued that this balancing of private and public benefits and costs will maximise the effectiveness of UFT.

In addition, it is essential to consider the issue of effectiveness of UFT in a door-to-door context, rather than just considering the impacts only within urban areas (so-called “last mile” impacts).  In the city of Chester in the UK, for example, a time window has been established by the local authority for morning deliveries between 08.30 and 10.30 to minimise potential conflicts with pedestrians in the medieval city centre during peak shopping hours; this appears to ignore the fact that the freight vehicles making these deliveries are likely to encounter heavy traffic congestion on routes into the city centre.

This paper continues by categorising private and public sector measures into broad functional types and then provides observations on each broad type using practical examples from around Europe that were observed during the course of a study of UFT carried out in 2011-12 for DG MOVE of the European Commission (MDS Transmodal and CTL, 2012). The study, which was based on literature review, site visits and interviews, was produced to assist the Commission in pursuing its ambitious policy goal of “essentially CO2-free city logistics in major urban centres”  by 2030 (European Commission, 2011).

2	Categorisation of measures

A categorisation of measures and practices is provided below to provide clarity and to aid analysis, although it has to be recognised that individual measures are rarely implemented at a local level in isolation but only as part of a mix of (more or less) complementary measures.  These categories are as follows:

- Regulatory measures;

- Market-based measures;

- Land use planning measures;

- Infrastructure measures;

- New technologies;

- Management and other measures.

Regulatory (or “command and control”) measures are essentially rules and prohibitions, supported by a control/enforcement system, that are designed, at least in theory, to control private activity for the wider benefit of society.  They are usually easier to implement for city authorities and also are likely to have a higher degree of acceptability among all stakeholders compared with, for example, market-based measures.  This is due to their more traditional nature and apparent equity.  They require, however, an accurate enforcement system to prevent possible infractions.  Examples include time windows for access to city centre freight deliveries and parking restrictions.

Market-based measures aim to modify the market prices of the goods whose production generates negative effects, with the objective of using the price mechanism to provide incentives to operators and their customers to change their behavior.  Examples include road pricing that affects the cost for freight operators of using the road network and congestion charging that affects the costs for freight operators of entering a designated area of a town or city.

Land use planning measures change the private use of space in urban areas for the public good, although this can only be achieved with a consistent policy over a long period due to the time it takes to change existing land use patterns.  Examples include planning regulations that allocate space to logistics activities in urban areas and that require the developers of new

office developments to provide for off-street loading/unloading bays for  freight vehicles.

Infrastructure measures are those that change or add to the physical transport network in, or on the approach to, urban areas.  Examples include the development of a network of on-street loading/unloading bays in cities for freight deliveries or the development of a rail- or water-connected distribution park on the outskirts of an urban area.

Technological measures and practices involve the application of innovative technology to the issues generated by UFT, such as the introduction of low emission vehicles (LEVs) for “last mile” deliveries or the application of Intelligent Transport Services (ITS) to allow the drivers of freight vehicles to respond to information on the current condition of the urban road network that is delivered to them in their cabs.

“Management and other measures” includes not only management measures that are implemented directly by private sector actors, but also all measures, whether implemented by private or public sector actors, that are not included in the other categories.  These are usually “bottom-up” measures that are “softer” in nature and often require some kind of collaboration between actors to achieve more effective UFT.  Examples include the consolidation of supply of UFT services where transport operators deliver their freight to an urban consolidation centre (UCC) on the outskirts for transfer to LEVs that make final deliveries with higher load factors in city centres.

- 3	Effectiveness of measures

3.1 Regulatory measures

Regulatory measures specifically for freight movements may be relatively easy to implement by city authorities, are often regarded as being acceptable by many stakeholders because of their traditional nature and are sometimes essential to ensure the safety of road users and residents/visitors and for the general good.  An example of essential regulation of freight activity would be a weight restriction on a bridge where the structure could be compromised by movements of freight vehicles and parking restrictions.

However, some other freight-specific regulatory measures appear to be implemented by city authorities without considering the implications on the overall effectiveness of freight activity.  For example, blanket weight restrictions for all freight within a large inner area of a city (not just a historic city centre) is likely to lead to freight acivity being carried out by low weight and low volume “white vans”, leading to an overall increase in environmental emissions and road congestion in the urban area.

Similarly, time windows for freight deliveries have often been established in the centres of cities to avoid conflicts between  pedestrians and freight vehicles.  This is usually a consequence of the desire to create a suitable environment for activities such as shopping and dining. However, where the time windows coincide with peak commuting hours (as for example in the city of Chester), they can lead to traffic congestion and are likely to lead to the poor utilisation of vehicles. While time windows are now “part of the scenery” in many urban areas, delivery time windows should therefore be made as wide as possible to facilitate economically efficient logistics and to avoid adding to road congestion in peak hours.

Similarly, the  geographic area covered by delivery time windows should not be so extensive that it is not possible for deliveries to be made outside time windows when using suitable cargo handling equipment to facilitate safe deliveries in close proximity to pedestrians. This means, in turn, that designated loading/unloading bays should be provided close to the areas affected by the time windows.   Ljubljana in Slovenia has a relatively generous time window (06.00 – 09.30) in operation for its pedestrianised historic city centre, but it also has a network of fifteen loading bays for freight close to the centre so that deliveries can still be made when the city centre is closed to freight traffic.  Furthermore, time windows should be extended wherever possible so that deliveries can be made outside peak periods of road congestion, as is possible in Ljubjlana.

Low Emissions Zones (LEZs) have been introduced in a number of European cities to limit access only to vehicles that meet certain emissions standards in an attempt to improve air quality.  There appears to be a lack of evidence on the effectiveness of LEZs, but they are likely to have some positive impact by reducing emissions from freight vehicles as freight operators are encouraged to renew their fleets and reduce their emissions without affecting logistical efficiency.  LEZs should not, however, only be applied to freight vehicles (as in London), as passenger cars are usually responsible for the greatest proportion of emissions in urban areas.

LEZs and, as far as possible,  other regulatory measures should also be harmonised at a regional level in the larger Member States and at a national level in the smaller Member States to allow operators to use their fleets as flexibly as possible and to reduce compliance costs.  The Netherlands has developed a national scheme to harmonise the rules applying to all LEZs that were implemented by different city authorities, with the objective of increasing the overall efficiency of the country’s city logistics.  Similarly, Emilia Romagna in Italy is seeking to harmonise access regulations for freight at a regional level to the centres of its many ancient cities.

Regulatory measures that could contribute to more effective UFT in European cities are summarized in Table 1.  They would involve making relatively straightforward amendments to existing regulations, rather than introducing additional regulatory measures, and would generally have a positive impact on the economic efficiency of UFT by reducing costs and reducing environmental impacts.

Table 1:  Summary of the effects of selected regulatory measures to increase the effectiveness of UFT

| MEASURE | ECONOMIC IMPACTS |  | ENVIRONMENTAL, HEALTH & SAFETY IMPACTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UFT operational costs | Road congestion | Air quality | GHG | Noise | Health & Safety |
| Extending time windows | Lower | Lower | Better | Lower | Neutral | Higher |
| Removal of vehicle access restrictions over extensive areas | Lower | Lower | Better | Lower | Neutral | Neutral |
| Harmonisation of regulations at regional or national level | Lower | Neutral | None | None | Neutral | Neutral |

Overall, regulatory measures are traditionally the most used in the context of UFT, but can be counter-productive if the objective is to increase the overall effectiveness of UFT.  The European freight industry frequently complains of strict regulatory measures introduced, in particular, to “close” city centres to freight traffic without any ex-ante evaluation of their impacts and without adequate consultation of the freight industry.

3.2 Market-based measures

Fiscal measures such as taxes and tolls aim to change the market price of the goods whose production generates negative impacts.  The price mechanism has a direct impact on the behaviour of the freight industry, as individual operators have to respond to changes in their costs to remain competitive.

Congestion charging, where a freight vehicle is charged a fixed amount to enter a specified area, has been introduced most famously in London and these schemes can provide  incentives for UFT operators to adopt more effective UFT practices.  The London Congestion Charge (LCC) has, for example, been cited as a reason  for the development of an urban consolidation centre (UCC) located outside the LCC to serve a number of retail outlets on Regents Street inside the LCC zone, as the consolidation of loads and the use of the same vehicles for a frequent shuttle service between the UCC and the shops reduces total costs and emissions.  In the long-run, however, congestion charges would not be required if a system of comprehensive “road pricing” was introduced in urban areas.

The internalisation of external costs (road pricing in the context of urban areas) is probably the most effective market-based measure in the long term to develop more effective UFT as it would involve charging freight vehicles for their use of the urban (and inter-urban) transport networks, taking into account the distance travelled, the type of vehicle (and its environmental and noise emissions), the time of day and its impact on congestion.  Any additional costs for the freight industry would have to be passed onto their customers, so that the shippers and receivers of freight consignments would also be incentivised to adopt strategies that would reduce costs (and externalities).  While the freight industry tends to argue that it would be unable to respond to these price signals that would be included in a road pricing scheme, freight operators and its customers would probably have a number of options to reduce costs:

- Deliveries could be made by LEVs;

- Load factors could be optimized to reduce the number of deliveries, perhaps through greater use of UCCs;

- Deliveries could be made outside peak hours, including during the night;

- Intelligent Transport Systems (ITS) and Information and Communication Technologies (ICT) could be deployed to an even greater extent to optimise routing

However, the implementation of road pricing in urban areas is likely to be beyond the technical and perhaps legal competence of almost all individual city authorities and would require coordinated action by the European Union and Member States through a comprehensive infrastructure charging scheme that includes the strategic networks as well as the urban road network.  Such a scheme is unlikely to be introduced in the short- to medium term, mainly for political reasons.

In the short- and medium term, though, the most practical market-based measures that can be adopted by city authorities to encourage more effective UFT are likely to be the use of differentiation to provide exemptions from regulatory provisions for UFT operators that adopt sustainable urban distribution practices, such as using LEVs for “last mile” deliveries.  For example, in Bristol LEVs that operate from the city’s UCC that serves the Broadmead Shopping Centre are exempt from the time window that restricts deliveries by other vehicles.  The provision of these indirect subsidies encourages sustainable UFT, but without requiring city authorities to provide subsidies from scarce resources that would also risk producing distortions in the market for providing freight services in urban areas.

Market-based measures that could contribute to more effective UFT in European cities are summarized in Table 2.

Table 2:  Summary of the effects of selected market-based measures to increase the effectiveness of UFT

| MEASURE | ECONOMIC IMPACTS |  | ENVIRONMENTAL, HEALTH & SAFETY IMPACTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UFT operational costs | Road congestion | Air quality | GHG | Noise | Health & Safety |
| Congestion charging | Higher | Lower | Better | Lower | Neutral | Neutral |
| Internalisation of external costs | Higher | Lower | Better | Lower | Neutral | Better |
| Indirect subsidies to “virtuous” UFT operators via differentiation | Lower (for some operators) | Lower | Better | Lower | Neutral | Neutral |

Market-based measures (and particulary road pricing) are often unpopular with freight operators because they are perceived erroneously as being “just another tax” and because of the lack of an alternative (which, in contrast, is available for passenger transport). However, if a pan-European “road pricing” scheme was carefully specified and implemented by the public sector at an EU and national level there are likely to to be very significant improvements in the effectiveness of UFT in Europe through the price mechanism.  If the scheme was designed in the appropriate way, it would also reduce the amount of regulation that would apply to freight activity (LEZs, for example, would no longer be required) and any direct and indirect subsidies to the freight industry could no longer be justified.

3.3 Land use planning measures

Effective planning of scarce urban space by city authorities should arguably take more account of the needs of UFT.  Land use planning in general has the potential to have a significant impact on the effectiveness of UFT in the long term, as long as suitable planning policies are adopted.  City authorities should adopt a holistic approach to land use planning that takes into account the demand for UFT generated by new developments as well as the needs of the freight industry.

Land use planning policies can, for example, make a major contribution to the development of adequate parking spaces for freight vehicles in urban areas.  In particular, the developers of any new office or retail development in urban areas should be required to plan for freight activity, with the development of off-street loading/unloading bays for new developments over a certain size.

Safeguarding of sites close to city centres that have access to rail and waterborne freight networks can allow the future development of more sustainable distribution services; London has followed a policy of safeguarding wharves on the River Thames to protect them from residential and office development.  In addition, so-called Urban Logistics Zones should be developed on the outskirts of larger urban areas to combine warehousing and lorry parking facilities in specific locations to generate economies of scale and reduce “logistics sprawl”.  Such a policy has been followed in the city of Bremen, for example, with the creation of its Guterverkehrszentrum (GVZ) which extends over some 220 hectares of occupied space.  In the case of the largest cities, intermodal rail freight terminals and/or inland ports should also be included in the logistics zones to facilitate the use of rail and waterborne freight services for medium to long-distance freight flows.

Land use planning measures that could contribute to more effective UFT in European cities are summarized in Table 3.

Table 3:  Summary of the effects of selected land use planning measures to increase the effectiveness of UFT

| MEASURE | ECONOMIC IMPACTS |  | ENVIRONMENTAL, HEALTH & SAFETY IMPACTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UFT operational costs | Road congestion | Air quality | GHG | Noise | Health & Safety |
| New developments with off-street loading/unloading facilities | Lower | Lower | Better | Lower | Neutral | Neutral |
| Developing Urban Logistics Zones | Lower | Lower, except in immediate vicinity of zone. | Better | Lower | Lower | Lower |
| Safeguarding of rail-connected & water-connected sites for future use | Lower | Lower | Better | Lower | Neutral | Neutral |
| Requiring large-scale distribution sites to be rail & water-connected | Lower | Lower | Better | Lower | Neutral | Neutral |

Land use planning measures can be very powerful in the medium to long term in fostering more effective UFT. However, these measures require the development of consistent planning policies over a long period of time and within specific national and local contexts.

3.4 Infrastructure measures

Rail, tram, inland waterways and underground schemes for the “last mile” transport of freight in urban areas are likely to to be successful only in niche markets.  A good example of a successful non-road last mile delivery scheme is provided by the “Beer Boat” in Utrecht, where beverages and other commodities are delivered by barge to canal-side bars and restaurants on the Oudegracht (“Old Canal”). However, this scheme has been successful because of the availability of an existing canal (providing “free” infrsatructure)  and the difficulties that are encountered by road transport operators in delivering to  the restaurants and cafes, which are below street level and can only be accessed by a narrow road with deliveries that have to be made down steep steps.

Nevertheless, heavy rail can be very effective in transporting freight over medium and long distances to rail-connected distribution parks/logistics zones on the outskirts of major urban areas for subsequent “last mile” delivery by road.  Inland waterways can play a similar role, particularly for construction materials and for high value freight, where an urban area is linked directly by an inland waterway to international gateways such as sea ports.

As a general rule road freight transport will remain the most important mode for last mile deliveries in urban areas due to its inherent flexibility. Infrastructure measures required to secure more effective UFT are, for all but the largest cities, likely to be limited to ensuring there is adequate provision for on-street loading/unloading bays where private off-street facilities are not available; Paris, for example, has a network of 15,000 on-street loading bays specifically for freight and the city has adopted guidelines to ensure that they are designed specifically for the needs of freight vehicles and their drivers.

Infrastructure measures that could contribute to more effective UFT in European cities are summarized in Table 4.

Table 4:  Summary of the effects of selected infrastructure measures to increase the effectiveness of UFT

| MEASURE | ECONOMIC IMPACTS |  | ENVIRONMENTAL, HEALTH & SAFETY IMPACTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UFT operational costs | Road congestion | Air quality | GHG | Noise | Health & Safety |
| Network of on-street designated loading & unloading bays | Lower | Lower | Better | Lower | Lower | Better |
| Development of rail- and/or waterborne connected logistics zones | Lower | Lower | Better | Lower | Neutral | Neutral |

The key infrastructure measure is likely to be the development of a network of on-street loading and unloading bays for freight (perhaps on a shared basis with other users) which should provide significant economic benefits in terms of lower UFT operator costs and reduced congestion costs.  The development of infrastructure for non-road modes should generally be focused on the development of large-scale rail and waterborne freight distribution parks on the outskirts of major urban areas rather  than on non-road schemes for “last mile” distribution that are unlikely to be economically or commercially viable and usually offer very poor value for money.

3.5 Technology measures

The widespread adoption of low emission and low noise vehicle and fuel technologies are likely to be essential for more effective UFT in the future as they reduce GHG emissions, improve air quality and facilitate the development of night-time deliveries. While the most promising alternative technologies for “last mile” deliveries in urban areas appear to be electric and hybrid technology, it is probably too early to make final and definite judgments about the most appropriate technology, given the amount of research and development work that is being carried out in the area.  Electric and hybrid vehicles have been introduced for freight operations in numerous European cities; for example, Rome has implemented a pilot freight transfer scheme where freight can be transfered to an electric vehicle, which is exempt from the time window regulations, at a designated transfer point for final delivery in the city centre.  A number of electric and hybrid freight vehicles, which are exempt from paying the Congestion Charge, are in operation in London.

ICT and ITS applications are already employed to increase the effectiveness of UFT operations by private sector road freight operators.  Given the clear cost benefits from the application of these technologies by large-scale freight transport operators in an urban setting it is very likely that the private sector will adopt appropriate new ICT/ITS applications when they become available in order to increase effectiveness and reduce costs.  Where the public sector at all levels can contribute to the effective implementation of ITS in urban areas is through the widespread implementation of systems that provide real-time data on the status of the road network, including the availability of loading bays for freight vehicles, in an interoperable way; this would allow road freight vehicles to operate in different cities without having to use different hardware and software. Tallinn in Estonia, for example, is developing an information system on the status of the city’s road network, which will provide information direct to driver’s mobile telephones; it is hoped that this system will help drivers follow the most appropriate routes through the city to access the strategic road network.

Finally, handling equipment fulfilling noise standards, in particular roll cages, may play an important role in increasing the acceptability of night deliveries.

3.6 Management & other measures

This category of measures includes not only management measures that are implemented directly by private sector actors in an attempt to reduce costs and provide a better service to their customers, but also a wide range of “softer” collaborative measures that are often implemented by partnership working between the public and private sector.

Urban Consolidation Centres (UCCs), which act as transfer points between in-bound freight on inter-urban movements and freight vehicles (with high load factors) for “last mile” deliveries perhaps by LEVs, have been established in many European cities with varying degrees of success.  The key issue for consolidation services in city centres is that they add cost to the door-to-door transport chain compared to direct delivery from a distribution centre to (say) a retail outlet. Italian cities have led the way in developing such services because of the heritage value of many city centres in the country and the very fragmented structure of the country’s logistics sector, which may  lead to lower load factors and more freight movements.  Some Italian cities (such as Padova) have chosen to provide  indirect subsidies to consolidation services to encourage their use, while more controversially, the city of Vicenza has made the use of its publicly owned consolidation services obligatory for deliveries into the city centre.  As a general rule, indirect subsidies are more effective at encouraging the use of private sector-led consolidation services, such as the “Cargohopper” service that is offered to the market by a private sector logistics provider in Utrecht;  the company uses an electric vehicle to make deliveries in the city centre, which enjoys an exemption from the time window.  Similarly, the availability of an exemption from the London Congestion Charge appears to have helped the development of a  private sector consolidation service for retail outlets on Regents Street.

Another approach to securing more consolidated deliveries in city centres is to secure greater collaboration in the procurement of deliveries, thereby influencing the demand for freight deliveries rather than the supply.  Transport for London has developed the concept of the Delivery Service Plan, where businesses that occupy offices in city centres actively manage the procurement of office supplies to reduce the number of deliveries they receive;  the businesses enjoy the benefits of lower procurement costs and negative externalities are reduced due to the lower number of freight movements in the city.  Similarly, in Gothenburg a number of organisations in the Lindholmen area of the city have joined forces to generate the critical mass of traffic required to justify a consolidated delivery service, so that almost all their deliveries pass through a consolidation point.

The development of the UFT measures by the public sector are more likely to be effective if they are developed within the context of formal transport plans. These plans, which could be called Urban Logistics Plans (ULPs), provide integrated plans for the development of  freight activity within the wider context of passenger mobility and all other urban policies.  Local Transport Plans in the UK, for example, are required to include policies specifically for freight, although the actual freight content is often quite limited unless there are very perceived to be specific freight-related issues in the area.  In Barcelona, however, the city has adopted a long-term strategy towards UFT with consistent political support and this approach has allowed the city to develop a consensus for the implementation of a number of freight-related measures over a ten year period.

Implementing night-time deliveries requires very close partnership working between the  private sector operators who wish to adopt night-time deliveries (often a supermarket chain and its logistics operators) and city authorities, who are representing a wide variety of stakeholder interests including residents whose sleep might be disturbed by the deliveries.  The experience of the supermarket chain Sainsburys at one of its south London stores, where it had to cease night-time deliveries due to complaints about noise from residents, highlights the importance of consultation with residents groups and their political representatives prior to implementation.

All these measures highlight the need for city authorities to consult closely with the freight industry and its customers, as well as other key stakeholders, while developing consistent long-term plans for encouraging effective UFT.  This allows the public sector to obtain a better understanding of the needs of the industry, but should also be supplemented by adequate data collection and ex-post evaluation of outcomes; in larger urban areas, the planning process should also involve an ex-ante evaluation of the proposed measures using modeling techniques.

Management and other measures that could contribute to more effective UFT in European cities are summarized in Table 5.

Table 5:  Summary of the effects of selected infrastructure measures to increase the effectiveness of UFT

| MEASURE | ECONOMIC IMPACTS |  | ENVIRONMENTAL, HEALTH & SAFETY IMPACTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UFT operational costs | Road congestion | Air quality | GHG | Noise | Health & Safety |
| Developing Urban Logistics Plans (ULPs) | Lower | Lower | Better | Lower | Lower | Higher |
| Developing Freight Quality Partnerships, involving effective consultation | Lower | Lower | Better | Lower | Lower | Higher |
| Indirect subsidies to support UCCs | Lower | Lower | Better | Lower | Lower | Neutral |
| Planning permission requirements for DSPs | Lower | Lower | Better | Lower | Lower | Neutral |
| Encouraging DSPs for existing businesses | Lower | Lower | Better | Lower | Lower | Neutral |
| Facilitating night-time deliveries | Lower | Lower | Better | Lower | Neutral | Neutral |

The summary highlights the number of relatively low-cost management measures that can be implemented by city authorities in partnership and consultation with UFT operators, their customers and other stakeholders to secure more effective UFT.  The key horizontal measure is the adoption of a process to develop an Urban Logistics Plan (ULP), in consultation with the freight industry and other stakeholders, that sets out an integrated strategy to secure sustainable urban distribution over the long term.  Other key measures are those that facilitate collaboration between receivers of freight in urban areas to procure consolidated deliveries, such as Delivery Service Plans (DSPs), and indirect measures to assist UFT operators to offer consolidated services for “last mile” deliveries.  Most of these measures focus on a partnership between the city authorities and the freight industry and their customers to develop improvements for the long term and do not provide “quick-wins.”  This, in turn, emphasises the importance of Urban Logistics Plans which can provide a long-term vision for local communities to develop measures and practices that secure more sustainable urban distribution.

4	Conclusions

Increasing the effectiveness of UFT could provide significant benefits to the freight industry and its customers through lower costs and increased efficiency, but would also provide benefits to society through reductions in externalities.  The freight industry is, as a general rule, continually seeking to reduce its costs in order to remain competitive but there remains an element of market failure because it is likely that only a proportion of the net externalities generated by freight movements to, from and within urban areas are reflected in the price charged by freight operators to its customers.  In the long term, therefore, a comprehensive system of road pricing at a European level including urban road networks could provide significant benefits. The technology already exists for its implementation. Such a system would provide appropriate price signals to the market and should lead to a reduction in other regulations and to the removal of all direct and indirect subsidies to the freight industry.

In the short- to medium term, there should be greater focus by city authorities on harmonising regulations at a regional or national level to reduce the regulatory burden on the freight industry and there should be greater focus on the implementation of ITS to assist freight operators as well as a structured approach to providing a network of on-street loading and unloading bays.

This will require a greater emphasis on planning for freight activity through so-called Urban Logistics Plans, which should be based on establishing knowledge on the freight industry through data collection and thorough industry consultation and ex-ante and ex-post evaluation techniques. Such an approach by city authorities would provide a consistent long-term strategic approach to freight policy in European cities, which would allow the freight industry to invest in equipment and operational practices to secure more effective UFT in the future.  It is only through private sector action, working in close collaboration with the public sector, that the European Commission’s objective of “essentially CO2-free city logistics in major urban centres”  can be achieved by 2030.

5	Bibliography

Alessandrini A., Delle Site P., Filippi F., Salucci M.V. (2012) Using rail to make urban freight distribution more sustainable. European Transport 50.

Allen J., Cherrett T., Browne M. (2012) Towards a more detailed understanding of urban road freight activity. Submitted to Research in Transportation Economics.

Bailly D., Meidinger M. (2010) The double trend of optimisation and social regulation of the postal activity in Europe. Paper presented at Rutgers Conference on Postal and Delivery  Economics, June 2-5, 2010, Porvoo, Finland.

BESTUFS II (Best Urban Freight Solutions) Project. Project site. http://www.bestufs.net/ (access on 12-6-2012).

BESTUFS II (2007) Good Practice Guide on Urban Freight Transport.

Boscacci F. e Maggi E. (2004) La Logistica Urbana – Gli Elementi Conoscitivi per una Governance del Processo. Polipress, Milano.

Browne M., Piotrowska M., Woodburn A., Allen J. (2007) Literature Review WM9: Part I - Urban Freight Transport, Green Logistics Project.

Browne M., Sweet, Woodburn A., Allen J. (2005) Urban Freight Consolidation Centres – Final Report.

Bruce M., Moore C., Birtwistle G. (2004) International Retail Marketing: a Case Study Approach, Butterworth-Heinemann.

CIVITAS Programme. Programme portal. http://www.civitas-initiative.org (access on 12-6-2012).

Comi A., Delle Site P., Filippi F., Marcucci E., Nuzzolo A. (2008) Differentiated regulation of urban freight traffic: conceptual framework and examples from Italy. Proceedings of the 13th International Conference of Hong Kong Society for Transportation Studies. Hong Kong, December.

CTL (2008) Indagine sulla Distribuzione delle Merci a Roma. Nell’ambito dello Studio di settore della mobilità delle merci a Roma.

Dablanc L. (2007) Goods transport in large European cities: difficult to organize, difficult to modernize. Transportion Research A, 41, 280-285.

Dablanc L. (2009) Freight Transport for Development Toolkit: Urban Freight. The International Bank for Reconstruction and Development/The World Bank

Dablanc L. (2010) Freight transport, a key for urban economies. Guidelines for practitioners. Proceedings of the Transportation Research Board 89th Annual Meeting, Washington DC.

Danielis, R., Rotaris, L., Marcucci, E. (2010). Urban freight policies and distribution channels: a discussion based on evidence from Italian cities. European Transport 46, 114-146.

D’Elia S., Festa D.C., Mazzulla G. (2004). La Distribuzione delle Merci nell’Area Urbana di Cosenza. Centro Editoriale e Librario dell’Università della Calabria.

European Commission (2007) Freight Transport Logistics Action Plan. COM(2007)607.

European Commission (2009) Action Plan on Urban Mobility. COM(2009)490.

European Commission (2011) White Paper. Roadmap to a Single European Transport Area - Towards a competitive and resource efficient transport system. COM(2011)144.

European Foundation for the Improvement of Living and Working Conditions (2005) Sector Futures – The Hotels and Catering Sector.

Greater London Authority (2006) Retail in London: Working Paper I - Service and Delivery Issues.

Hay G. (2005) Hotels and catering sector: visions of the future.

ICSC Europe (2009) Retail Space Europe. Yearbook 2009.

IMAP (2010) Retail Industry Global Report.

INRETS (2010) Good Practices Analysis. SUGAR Project, Deliverable 3.3.

ISIS & Price Waterhouse Coopers (2010) Study on Urban Access Restrictions. DG MOVE, European Commission.

Issenmann, J., Delaitre, L., Danard, J., De Barbeyrac, C. (2010) Integrating goods in public transport: the case of Paris. Proceedings of 12th World Conference on Transport Research, Lisbon, Portugal.

ITA Consulting, WIK-Consult (2009) The Evolution of the European Postal Market since 1997. Study for DG Internal Market and Services, European Commission.

LUISS Business School, AICAI (2010) The Business Model of International Express Couriers. From Value Chain to Policy Indications.

Maes, J., Vanelslander, T. (2010) The use of rail transport as part of the supply chain in an urban logistics context. Proceedings of 12th World Conference on Transport Research, Lisbon, Portugal.

Maggi E. (2007) La Logistica Urbana delle Merci. Aspetti Economici e Normativi. Polipress, Milano.

Makay D., Sobrino Maté I., Vallini M., Waravka A. (2009) HOTREC Hotels, Restaurants and Cafes in Europe. Some 60 European Union measures affecting the European hospitality industry.

Martinez M., Navarro C., Ripsam T. (2007) Big Impact in a Small Format: A New Wave in Retail. Booz Allen Hamilton.

MDS Transmodal and CTL (2012) Study on Urban Freight Transport. Final Report for DGMOVE of the European Commission.

Ministero dei Trasporti (2007) Realizzazione di un’Indagine/Studio sul Settore dell’Autotrasporto in Merito alla Raccolta e alla Distribuzione delle Merci in Area Urbana. IZI e CSST.

Ministero dei Trasporti (2010) Piano della Logistica - Morfologia dei Flussi Logistici Internazionali. “Feelings & Insights” del Sistema Logistico Italiano. D’Appollonia.

Monoprix (2008) Rapport Développement Durable 2008.

Munuzuri J., Larraneta J., Onieva L., Cortés P. (2005) Solutions applicable by local administrations for urban logistics improvement. Cities 22, 15-28.

OECD (2003) Delivering the Goods. 21st Century Challenges to Urban Goods Transport.

Ogden K.W. (1992) Urban Goods Movement: a Guide to Policy and Planning. Ashgate Publishing Company, Cambridge.

Persson G., Jensen L., Engebrethsen E. S., Flygansvær B. (2008) Distribution Networks in Transition.

Piotrowska M., Browne M., Briggs D., Richardson, Wainwright (2009) The Scope for Sustainable Distribution and Servicing in the Regent Street Area of Central London.

Ponce-Cueto E., Carrasco-Gallego R. (2009) Distribution Models for Logistics in HORECA Channel.

Purcell K. (2008) Introduction to the HORECA Sector. European Agency for Safety and Health at Work.

Quak H.J. (2008) Sustainability of Urban Freight Transport Retail Distribution and Local Regulations in Cities.

Quak H.J. (2011) Urban freight transport: the challenge of sustainability. In: Macharis C., Melo S. (eds) City Distribution and Urban Freight Transport: Multiple Perspectives. Edward Elgar, Cheltenham.

Regione Emilia Romagna (2003) Linee Guida per una Strategia Regionale di Logistica Urbana. Quaderni del Servizio Pianificazione dei Trasporti e Logistica.

Regione Emilia Romagna (2005) Logistica Urbana a Bologna. Elementi per un Progetto. Quaderni del Servizio Pianificazione dei Trasporti e Logistica.

Richardson E. W. (1975) Growth Centers, Rural Development, and National Urban Policy: A Defense. In: Friedmann J., Alonso W. (eds.)  Regional Policy: Readings in Theory and Practice. Cambridge: MIT Press. 97-132.

Robinson A. M., Mortimer P. N. (2004) Rail in urban freight. What future, if any? Logistics & Transport Focus.

Robinson A. M., Mortimer P. N. (2004) Urban freight and rail – The state of the art. Logistics & Transport Focus.

Russo F., Comi A. (2011) Measures for sustainable freight transportation at urban scale. Journal of Urban Planning and Development 137(2), 142–152.

Schoemaker, J., Allen, J., Huschebek, M., and Monigl, J. (2006) Quantification of Urban Freight Transport Effects I. BESTUFS Consortium.

Stantchev D., Whiteing T. (2006) Urban Freight Transport and Logistics. An Overview of the European Research and Policy.

Thompson R.G., Taniguchi E. (2001) City logistics and freight transport. In: Brewer A.M., Button K.J., Hensher D.A. (eds) Handbook of Logistics and Supply-Chain Management. Pergamon, Amsterdam.

Transport & Travel Research Ltd. (2008) Out of Hours Deliveries in Central London.

Transport for London (2007) London Freight Plan. http://www.tfl.gov.uk/microsites/freight/documents/London-Freight-Plan.pdf (access on 12-6-2012)

Transport for London (2009) A Pilot Delivery Servicing Plan for TfL’s Palestra Offices in Southwark.

Whiteing A., Browne M., Allens J. (2003) City Logistics: The continuing search for sustainable solutions. In Waters, D. (ed.), Global Logistics and Distribution Planning, Kogan Page.

CTL - Centro di ricerca per il Trasporto e la Logistica, University of Rome “La Sapienza”, via Eudossiana 18, 00184 Rome, Italy. andrea.campagna@uniroma1.it

Address as above.  paolo.dellesite@uniroma1.it

Address as above. francesco.filippi@uniroma1.it

MDS Transmodal Limited, 5-6 Hunters Walk, Canal Street, Chester CH1 2NH, UK. chris.rowland@mdst.co.uk

As shown by the introduction of the Maut in Germany and Austria on the strategic road network.